Passive

Default state entered upon power on, Clean complete, Brew complete or Reset command received

All valves = default positions;

Flow control valve = default position;

PWM off;

Pump off;

Heater off;

Translates and sends sensor Info (Background function, always running)

Must be in Passive state for WPS to function

If WPS_Button ==1

State = WPS;

Control over active recipe / config	*Do we only want to be able to change these while in 						Passive? Maybe we want the ability to adjust the hop timing 					mid brew, but would these sorts of changes potentially 						break our code...?

If brew

State = Safety Check;

Else if Clean

State = clean;

Else if Manual
	       State = Manual;

WPS

Run WPS routine;

State = Passive;

Clean

Accessible from "Passive" upon user request

Pause / resume functionality

Reset immediately sends to "passive state"

Pre fill or Fill from tap options

If tap, fill with xxL (Flow rate measurement)

Translates and sends sensor Info

Rinse

Pump water around system for X minutes and cycle valves to include all lines

On completion prompt user to move O/P hose to sink before draining

Pre-set Clean

Automatically inputs pre-set temperature, volume and times into the manual fields and sends command to micro.

Manual

Multiple user configurable stages with volume, temperature and time inputs
Drain prompts sent to user at correct times

Return to "Passive" state upon completion

Manual

Accessible from "Passive" upon user request

Pause / resume functionality

Reset immediately sends to "passive state"

Inputs: Valve states, Desired flow %, Desired Temp, Pump on/off, Heater on/off, Heater power %, Timer, input volume, Update, Finish

If Update
	Set all control variables to user defined parameters;
	    If Time == Timer
		Set all control variables to default;
	Else If (flow rate volume = input volume && flow rate volume != 0)
		Set all control variables to default;

Else if Finish
	State = Passive;

Safety Check

Accessed once user presses "brew"

If (Main config == 01)

{

Kettle_Check = Total recipe volume < (Kettle volume - small safety margin constant)

Mash_Check = (Grain weight to volume conversion + mash water volume) < (Mash Tun 	volume - small safety margin constant)

}

Else

{

Kettle_Check = Total recipe volume < (Kettle volume - small safety margin constant)

Mash_Check = (Grain weight to volume conversion + mash water volume) < (Kettle 		volume - small safety margin constant)

}

If (Kettle_Check && Mash_Check)

State = Mash;

Else

Return Kettle_Check, Mash_Check;

State = Passive	//give appropriate error message

Mash

Accessed from "Safety Check" state

Pause / resume functionality

Reset immediately sends to "passive" state

Main Inputs: Main config, Volume, Strike temp, hose/prefilled

If Hose begin

Valve 1 -> external connection;

If Flow rate volume == mash volume

Valve 1 -> kettle;

End;

Else

Prompt user to fill kettle with recipe defined volume of water

Preheat and recirculate until "strike temp" reached and
prompt user to add water chemistry additions

If Main Config == 01

Valves -> mash tun;

Else

Prompt user to insert BIAB and grain

Stage inputs: Time, Temp, flow rate

Stage 1

.

.				Close valve 3 if float 1 == 0 at any stage

.				PID for flow rate...?

Stage n

Proceed to "sparge" state on completion

Sparge

Accessed from "Mash" State

Pause / resume functionality

Reset immediately sends to "passive" state

Inputs: Main Config, Tap / tank, Heating method, volume, temp

If Main Config == 11 //BIAB no sparge

State == boil;

Else

If Heating method == 0

Heater off;

Prompt user to add preheated water to sparge tank and make

Chemical adjustments;

Valve 1 -> external connection;

Pump on;

Flow Rate control valve = pre defined sprinkle setting;

If Flow rate volume == sparge volume

Pump off;

Valve 1 -> kettle;

Wait x time for mash tun / BIAB to drain; // hard coded time

State == boil;

Else If Tap

Prompt user to connect external connection to tap;

Pump off;

Valve 1 -> external connection;

Heater on max;

Flow Rate control valve adjusted to hit sparge temp;  // PID controlled

If Flow rate volume == sparge volume

Heater off;

Valve 1 -> kettle;

Wait x time for mash tun / BIAB to drain; // hard coded time

State == boil;

Else If tank

Prompt user to full tank and make water adjustments;

Valve 1 -> external connection;

Pump on;

Heater on max;

Flow Rate control valve adjusted to hit sparge temp;   // PID controlled

If Flow rate volume == sparge volume

Heater off;

Pump off;

Valve 1 -> kettle;

Wait x time for mash tun / BIAB to drain; // hard coded time

State == boil;

Boil

Accessed from "Mash" State

Pause / resume functionality

Reset immediately sends to "passive" state

Inputs: Time, adjunct time, Active cooling, water/wort

Valves -> kettle;

Pump on;

Flow rate control valve -> pre defined setting;

If time == adjunct1 time + 3 min

Prompt user to add adjunct1 in 3 min;

If time == adjunct1 time

Prompt user to add adjunct1;

.

.

.

If time == adjunct n time + 3 min

Prompt user to add adjunct n in 3 min;

If time == adjunct n time

Prompt user to add adjunct n;

If (time == 10 && Active cooling)

If Wort

Prompt user to configure for wort cooling;

Wait for user confirmation;

Open Valves to chiller and pump through to sterilise;

Else if water

Prompt user to configure for wort cooling and to insert chiller into boiling wort;

If time == boil time

Pump off;

Heater off;

State == Cooling;

Cooling

Accessed from "Boil" state

Pause / resume functionality

Reset immediately sends to "passive" state

Inputs: Target temp, Active cooing, Wort/water recirc

If Active cooling == 0

Prompt user "press to proceed to transfer stage"

Wait for user input;

State = Transfer;

Else If wort

Pump on;

If temp = target temp;

Pump off;

State == Transfer

Else if temp can not reach target temp

Error message;

If user confirmation

State = Transfer;

Else    //water

Prompt user to turn on tap;

If temp = target temp;

State == Transfer

Else if temp can not reach target temp

Error message;

If user confirmation

State = Transfer;

Transfer

Accessed from "Cooling Stage" state

Pause / resume functionality

Reset immediately sends to "passive" state

Inputs: Pumped transfer

If Pumped transfer = 0

State = Passive;   //brew complete

Else

Prompt user to configure the transfer;

Wait for user confirmation;

Pump on;			*Measure and display transferred volume?

If flow rate == 0

Pump off;

State = Passive; //brew complete