Make an account to log in

Register device

Configuration warnings e.g input recipe 30l, but config is 25l boiler. Or mash volume + grain wight won’t fit in mash tun. App can adjust 1l of water absorbed for kg of grain.

Save recipes

Save your device connection

Set up and save brewery configuration – main boiler volume, mash tun? Volume. Inline sparge heating? (or pre heated sparge water). Tap connection? If not then will manually fill main boiler at start and sparge tank. Cooling method – needs elaboration.

Other configs – BIAB vs sparge etc.

Settings – metric/imperial

Settings – lead time for any user interaction required notifications e.g. hops additions e.g. 5 minutes before interaction required.

Brew

Step 1. Confirm brewery config

Load recipe from recipes, auto populates input fields and confirm

Main info

Mash water volume – e.g. 30l brew = 15l

Sparge water volume required – e.g. 30l brew = 20l (depends on grain weight)

Grain weight – needs to know this to help with calculating water loss to grain

Mash stage info

Strike Temperature (temp for when to add grain) temp for mash water. E.g. 73°c for ~65 ° mash due to temp loss

Mash Temp ( temp to maintain when recirc mash) e.g. 65°C

Mash Time Stage 1 e.g. = 60 minutes

Add Mash Stage (plus sign)

Mash Time Stage 2 e.g. 30 minutes

Mash Out (raise temp before draining) Temp e.g. 75°C, leave as 0 for skip this step. Or same as mash temp – auto populates to last stage mash temp and can change if desired

Mash Desired pH e.g. 5.4 – no automation adjustment but can push notification if out of limits e.g. pH too low/high

pH acceptable range %. E.g. send me a notification if pH is out by this much – or possibly hard code?

Sparge stage info

Sparge Temperature e.g. 75 °C

Max Sparge Flow Rate – but temperature takes priority – more appropriate for non RIMS

Possibly replicate the mash pH stuff here. For pump input pH sensor

Boil stage info

Boil time e.g. 60 minutes

Adjunct (hops) addition time 1 e.g. 25m

Add adjunct addition time - plus sign

Adjunct addition time 2 e.g. 11m

Not recipe info:

Adjunct Notifications include what to add, require confirmation of adding in the brew in progress page. Push second notification at addition time if ignored

(error check any adjunct add time is less that boil time)

Notification user to Connect cooler(dependant on brewery config settings) at the half way of boil point, requires user confirm before moving onto next stage.

Cooling Stage

(Prompt user to start cooling e.g cooler in ice water or turn on tap depending on config. ) – not recipe info

Target temperature to cool to e.g. 25 °C

End of recipe entering, button to start brew. Button to save recipe. Button to exit.

Recipe Page

New recipe – same as brew enter recipe shit. Minus the start brew button

View save recipes. List of saved recipes, displays like new recipe fields but not editable. At the bottom has edit button. Exit button. Brew this recipe button.

Scale factor something – ability to scale batch in a recipe.

Cleaning

Quick clean

Full clean

Brew/Work in progress page

Distill page (similar to recipe)

Equipment config like brew setup

Flow rates, temperatures etc

Remembers previous settings/recipe

Notes:

Some kind of brew in progress page, Current stage, time left, sensor info etc. sparge pH

Push notifications to phone, various things e.g. stage complete, error, user interaction required.

I want to view current brew progress/info

Use the app within range of my whole house.

Be notified of things

Non-functional: easy navigation of app,

Need to add: cancel/stop/modify/pause/override during brew – button in brew in progress screen

Add a manual mode to menu